%%Simulation study of BPSK in presence of Noise

clc %for clearing the command window

close all %for closing all the window except command window

clear all %for deleting all the variables from the memory

Fs=1000;

%L=Fs;

L=100;

%t=0:0.1:10;

T=1/Fs;

t = (0:L-1)*T;                % Time vector

%t=0:0.001:1; % For setting the sampling interval

fc=input('Enter frequency of Carrier Sine wave: ');%100Hz

fm=input('Enter Message frequency : ');%10Hz

amp=input('Enter Carrier  Amplitude:');%2V

SNR=input('Enter SNR in db');%enter value from 5 db to 30 db see the variation

c=amp.*sin(2*pi*fc*t);% Generating Carrier Sine

subplot(5,1,1) %For Plotting The Carrier wave

plot(t,c)

xlabel('Time')

ylabel('Amplitude')

title('Carrier')

m = sign(sin(2*pi*fm*t)); % Generates -1 and +1 like square()

subplot(5,1,2)

plot(t,m)

xlabel('time')

ylabel('amplitude')

title('Message Signal')% Sine wave multiplied with square wave in order to generate PSK

x=c.*m;

subplot(5,1,3) % For Plotting PSK (Phase Shift Keyed) signal

plot(t,x)

xlabel('t')

ylabel('y')

title('PSK')

Noisy = awgn(x, SNR);

subplot(5,1,4) % For Plotting PSK (Phase Shift Keyed) signal

plot(t,Noisy)

xlabel('t')

ylabel('y')

title('Noisy PSK')

NFFT=2^nextpow2(L);

S=fft(x,NFFT)/L;

f = Fs/2*linspace(0,1,NFFT/2+1);

% Plot single-sided amplitude spectrum.

subplot(5,1,5);

plot(f,2*abs(S(1:NFFT/2+1)))

title('Single-Sided Amplitude Spectrum of y(t)')

xlabel('Frequency (Hz)')

ylabel('|S(f)|')